- Purpose

The purpose of this procedure is to provide for a system and instructions, and to assign responsibilities for establishing and maintaining Device Master Records (DMR).

- Scope

This procedure applies to technical/engineering documentation for each type or model of medical device or system.

- Definitions

| Device Master Record | Device Master Record is an index (i.e., a table of contents) of device-specific documentation required for procurement of materials and components, manufacture, and evaluation of a finished medical device. |
| --- | --- |

- Responsibilities

| Project Manager | - Complies with the policy and procedure for all design development projects. May delegate this activity to another design - Ensures the most current version of this document is used when referenced. |
| --- | --- |
| QA | - Ensures proper training before using this policy or procedure. - Participates in the development, reviews and/or approval of all design control documentation. |

- PROCEDURE

  - General

    - DMR is also a current record and status of the physical configuration of the device.

    - Unique DMR is established for each type or model (family) of medical device manufactured by EPIC Research & Diagnostics, Inc. (EPIC).

  - Documents that may be included in DMR

For each type or model of medical device the DMR may include the following types of documents:

    - Device Specifications (DS):

- product trade and common names;

- intended uses;

- performance characteristics and theory of operation;

- physical characteristics;

- environmental limitations, product stability and storage requirements;

- user safety characteristics;

- component, subassembly and assembly drawings and specifications;

- bills of materials;

- wiring and piping diagrams;

- software specifications.

    - Manufacturing Process Specifications (MP):

- process flow charts;

- process/assembly lines diagrams;

- equipment, tools, molds;

- manufacturing environment specifications;

- setup procedures;

- operator instructions;

- equipment maintenance procedures;

- validation reports for special processes;

- sterilization specifications, procedures, and validation reports;

- blank work orders, nonconforming product/process forms, and other reporting forms.

    - Quality assurance procedures and specifications (QA):

- quality system operational procedures (QOP);

- process control specifications/charts;
control plans, instructions and acceptance criteria for incoming, in-process, and finished device inspection and testing;

- procedures and acceptance criteria for the verification of packaging, labeling, installation, and servicing activities;

- blank work order forms for recording inspection/testing activities, traceability, and other data for device history records;

- device release review/evaluation checklists.

    - Packaging and labeling specifications (PL):

- package drawings and specifications;

- filling/packaging procedures;

- label/labeling drawings;

- instruction manuals.

    - Installation, maintenance and servicing specifications (IMS):

- installation, specifications and instructions;

- maintenance instructions;

- servicing specifications and manuals.

    - Not all types of documents specified in this section are relevant for each type or model of device. For a particular device, representatives from the appropriate areas including: project management, Engineering, Operations, and Quality will decide jointly which specific documents are to be included in the DMR. Edit to accurately assign the responsibilities for determining the contents of the DMR.

  - Format of the DMR Index

    - DMR is typically maintained on computer in the form of an Excel spreadsheet. The model of the spreadsheet is provided in Device Master Record Index.

    - To determine the effective date, revision level, and distribution of a document included in the DMR, the DMR Index is cross-referenced with the QA records where document status information is maintained (refer to EPIC’s Documentation System Policy).

  - Review, approval and change control

    - The initial DMR Index is approved by the design review board as documented by signatures on an approval cover sheet.  Once approved by the design review board, the initial DMR is placed under document control according to EPIC’s Documentation System Policy.  Any subsequent addition or deletion of documents from the index, are reviewed and approved by the QA Manager.

    - The review and approval of the DMR Index is for ensuring the completeness and relevance of the index, rather than for the correctness of the individual DMR documents. Individual documents in the index are independently reviewed and approved by their issuing authority, in accordance with EPIC’s Documentation System Policy.

    - DMR review, approval, change control, distribution control, retention of obsolete copies, and other document control requirements are the same as apply generally to controlled documents, as specified in EPIC’s Documentation System Policy and Records Retention procedure .

- ASSOCIATED DOCUMENTS

QA-005 Documentation System Policy

OPS-010, Records Retention

EGF-013, Device Master Record Index

- Attachments

N/A

Document Revision History

| Change Order Number: | Version Number: | Description of Change | Date: | Updated by: |
| --- | --- | --- | --- | --- |
| CC-120601-01 | 000 | New Document | 6/12/12 | A. Miller |